Coverage-Combustibility Gap

A physics-aware fusion of Mireye ignition risk and USFA fire-response capacity for WUI capital planning

# 1. Technical Approach

## Problem & Question

Which rural / wildland-urban interface (WUI) housing clusters are both physically primed to ignite and furthest from the fire-response capacity meant to protect them — and where is that gap widening? The output is a ranked, physics-justified shortlist for a county emergency manager or rural fire district board, not a generic risk map.

## Data Fusion

Mireye (ignition-side, static/structural fields, hex-aggregated):

- Slope and aspect (topography)

- Fuel / vegetation classification proxy

- Construction-era / roof-material proxy → thermal-inertia stand-in

- Wind exposure (NOAA climatological normals where Mireye lacks a wind field)

- Housing density; distance to nearest surface water source

USFA National Fire Department Registry (response-side, the actual second dataset that fuses with Mireye):

- Station locations, apparatus type/count, staffing model — free, public, geocodable

- Census Bureau geocoder used to fill coordinate gaps in the registry

The join point — a hex-level score that measures ignition risk against response capacity — exists in neither source alone. That is the fusion.

## Physics Enrichment

Ignition Propensity Score (IPS): a Rothermel-inspired weighted composite of fuel load, slope, wind exposure, and the thermal-inertia proxy, each normalized 0–1.

Response Capacity Score (RCS): drive-time to nearest adequately staffed station (via OSRM, static routing — no live traffic dependency) evaluated against NFPA-1 required fire flow for local housing density and construction class.

Coverage-Combustibility Gap (CCG) = IPS × (1 − RCS, normalized). Multiplicative by design: a fire-resistant hex doesn't matter how underserved it is, and a well-covered hex doesn't matter how flammable it is — only the intersection is the planning problem.

## Agentic / MCP Architecture

Four tools, deterministic math kept out of the LLM:

get_hex_physics(hex_id)         → Mireye enrichment fields

get_nearest_stations(lat,lon,n) → USFA station data + drive time

compute_gap_score(hex_id)       → IPS / RCS / CCG + reasoning trace

draft_capital_brief(hex_id)     → LLM turns trace into a planning memo

The LLM orchestrates and narrates; it never computes the score itself — that keeps the core USP (physics-grounded, non-hallucinated numbers) intact.

## Validation

Top 15–20 ranked CCG hexes checked for overlap against CAL FIRE Fire Hazard Severity Zones and, where available, ISO PPC ratings — both free and public. Reported as correlation against known ground truth, not a pass/fail claim.

## Stack

- Backend: Python — scoring module + MCP server

- Frontend: deck.gl (native H3 hex layer) for the ranked map view

- No live weather, live traffic, or dispatch integration — everything static/batch, which keeps Mireye credit spend low and removes demo-day flakiness

## Phase 2 (Stretch Goal): Active Fire Response

Extends the same physics engine from preemptive risk to an active-fire scenario, without requiring a live incident or unverifiable dispatch/broadcast integrations.

- Spread simulation: replay a real historical WUI fire (CAL FIRE incident archive — known ignition point, recorded weather/wind, documented final burn perimeter) through a Rothermel-based spread model driven by Mireye terrain, fuel, and wind fields

- Validation: score the simulated perimeter against the real, documented perimeter — this is the ground truth the preemptive phase lacks, and it directly answers 'test against reality' for the active-fire half

- Apparatus recommendation (decision-support, not live dispatch): NFPA-1 fire-flow math against the simulated fire's scale, output as a recommended apparatus count and nearest-station list — a human dispatcher acts on it, the agent does not autonomously call anything

- Citizen alert geofence (artifact, not live broadcast): agent generates a physics-based danger polygon (not a generic radius) and outputs a CAP 1.2-formatted alert payload — demoed as a generated artifact, not an actual broadcast

This keeps every physics claim from the original two-scenario vision intact while removing the two pieces that cannot be tested or built by a small team in the hackathon window: live dispatch integration and live public alert broadcasting.

# 2. Progress Made So Far

# 3. Future Plans

- Confirm exact Mireye field availability and per-hex credit cost via GET /v1/meta/fields; lock the field list before scaling beyond the pilot county

- Finalize IPS/RCS weighting against published Rothermel sensitivity literature rather than arbitrary defaults

- Complete USFA registry geocoding pass and station apparatus/staffing normalization for the pilot region

- Run the CAL FIRE FHSZ / ISO PPC correlation check on the top-ranked hexes and report validation results

- Build the deck.gl dashboard: hex map, click-through reasoning trace, and drafted capital-planning brief per hex

- Pilot with one county emergency manager for qualitative feedback on the brief's usefulness before wider rollout